Environmental and Social Management Plan (ESMP) Checklist

For 360 kWe / 435.6 kWp Solar Power Plant of

Kayapınar Municipality

Date of Issue: August, 2025

Document History

| Revision | Submitted to | Issue Date | Revision Details |
| --- | --- | --- | --- |
| v1 | ILBANK | 26.03.2025 | Draft |
| V2 | ILBANK | 23.06.2025 | Draft |
| V3 | ILBANK | 26.08.2025 | Draft |

This document has been prepared by ÇA Engineering Company.

Environmental and Social Management Plan (ESMP) Checklist

# Part 1: General Subproject and Site Information

| 1.a) General |  |
| --- | --- |
| Associated ILBANK Project | Türkiye Public and Municipal Renewable Energy Project (PUMREP)Türkiye Public and Municipal Renewable Energy Project (PUMREP) |
| International Financial Institution (IFI) Financing the Project | The World Bank |
| Project’s E&S Risk Classification according to WB ESF (2018) | ModerateModerate |
| Subproject Title | Kayapınar Municipality 360 kWe / 435.6 kWp Solar (Photovoltaic) Power Plant (SPP) |
| Sub-borrower Name | Kayapınar Municipality |
| Responsible ILBANK Regional Directorate (RD) | Diyarbakır Regional Directorate |
| Subproject’s E&S Risk Classification according to ILBANK ESMS (2023) | ModerateModerate |
| Subproject Location | Province: Batman District: Gercüş Neighborhood: Yenice Parcel/Block no: 21/106 |
| Scope of Subproject and Activity (in case of any changes of the subproject please fill Appendix-12 and submit to ILBANK) | Technology (e.g. Photovoltaic, monocrystalline, polycrystalline, thin film, bi-facial, tracking system, etc. ): Monocrystalline Installed power: 435.6 kWp Connection power: 360 kWe Annual electricity generation: 771 MWh/ year Construction Duration: 4-5 weeks Operation Duration (Economic life of the Plant): 25 years Number of Construction Workers (at peak, including contractors and subcontractors): 15 workers Number of Operations Workers (at peak): 2 workers Planned accommodation: Off-site (rented houses in nearby settlements) |
| Energy Transmission Line (ETL) | Grid connection: For the sub-project, approximately 40 m ETL will be made and the pole to which the system connection will be made is approximately 5 m away from the power plant site. After the ETL transformer, 20 m underground and then 20 m overhead line will be made and the network connection will be made with a loose connection. A map/visual clearly showing the route and pole locations of the energy transmission line (ETL) has been added to this document (See Figure 1). The entire line and the location of each pole are clearly marked on the map. It is clearly stated that the route does not pass through any parcel subject to private ownership and this is confirmed by the relevant cadastral data. The ownership status of the areas where the poles will be located has been analyzed and it has been determined that these areas are within public land and therefore do not require expropriation, allocation or any other land acquisition process. In this context, there will be no intervention in private property rights for the construction of the energy transmission line. Status of transformer station: Building Energy transmission line (ETL):20 m underground and 20 m overhead. Figure 1. Sub project ETL An ETL will be constructed within the scope of this subproject. The technical information on the ETL is provided below: Transformer station: Connection will be made to pole number 7245050. Length of the route: 40 m Voltage level: 31.5 (kV) Number of ETL towers (pylons): 2 pylons Total footprinte area per each ETL tower: 4.5 m2 Status of land acquisition: Allocation (See Appendix-3) |
| Access Roads | There is currently an access road to the project site. The access road is asphalted and is sufficient for the transportation of equipment to the site. Therefore, there is no need for a new road construction or road improvement. |
| Other Associated Facilities: Are there any other associated facilities that are not funded as part of the Subproject and are (a) directly and significantly related to the Subproject, (b) carried out, or planned to be carried out, contemporaneously with the Subproject, and (c) necessary for the Subproject to be viable and would not have been constructed, expanded or conducted if the Subproject did not exist? | No |
| Existing Permits | There are decisions from the Planning and Urban Development Directorate regarding the subproject that an SPP conformity letter, numbered E-48127494-754-29875 and dated 06.11.2024, and an SPP Building permit, numbered E-48127494-754-32752 and dated 04.03.2025, is not required. In addition, according to the official letter of Batman Special Provincial Administration, Planning and Urban Development Directorate, dated 06.11.2024 and numbered E-48127494-754-29875, there is a decision that there is no problem in terms of zoning. See Appendix-2. |
| 1.b) Site Description |  |
| Subproject Area | The parcel has an area of 20,000 m2 and 4,650 m2 will be used for the subproject area. Number of parcels to be used: 1 Total title deed area of the parcel: 20,000 m2 Total area to be used by the Subproject (within the fence area): 4,650 m2 |
|  | Appendix-1: Site Map Appendix-4: Photolog |
| Who owns the land? Since when? | The land designated for the implementation of the sub-project was officially allocated to Kayapınar Municipality by the General Directorate of National Real Estate in 2024. Appendix-3: Title Deeds |
| Land Registry Type according to Title Deed (agricultural, pasture, vacant, etc.) | Raw Earth |
| Current Land Use (are there any formal or informal agricultural users, herders, etc.) | Currently, no livestock grazing or animal husbandry activities are carried out within the sub-project site. The area contains naturally grown oak and hornbeam trees. However, these trees will remain outside the fenced area of the sub-project site, so no relocation or cutting will be necessary. Approximately 50 meters west of the sub-project site, there is a structure where the groundwater level is measured. Images of the structure are given in Appendix-4. The previously unused water well located within the parcel boundaries was excluded from the area where the solar power plant will be installed as a result of the parcel arrangement. Allocation procedures for the relevant properties were carried out within the framework of current legislation and Presidential Circulars (2018/8, 2022/15, 2024/7), and the allocation document for the well was issued on Appendix-2. |
| Other Nearby Facilities and Activities Are there other industrial or commercial activities operated/operating or planned by the Sub-borrower itself or other public or private third-parties in the vicinity of the Subproject or its components/associated facilities? | There are no other industrial or commercial activities operated/run or planned near the Sub-Project or its components/associated facilities by Kayapınar Municipality or other public or private third parties. |
| Area of Influence | The Area of Influence (AoI) for the sub-project is determined based on the anticipated environmental and social impacts during the construction, operation, and maintenance phases of the subproject, such as local ecosystems, nearby communities, and critical infrastructure. The access route passes in front of the neighborhood and does not intersect with sensitive structures such as health and school. Therefore, no traffic impact is expected during transportation and equipment transportation to the site. Within the scope of sub-project activities; AoI for the sub-project was primarily determined based on technical assessments, including dust and noise dispersion modeling, air quality and traffic analysis, and proximity to sensitive receptors. Consultations with local people and the mukhtar of the Yenice neighborhood were also conducted during the site visit to gather supplementary information on potential concerns and perceptions regarding the sub-project. Additionally, details regarding dust emissions and environmental noise calculations to occur during sub-project activities are provided in Appendix-11 The closest sensitive receptor to the subproject site is the residences located 200 meters away. During the field visits, 3 people living around the subproject area and the mukhtar of Yenice neighborhood were interviewed, and the environmental and social risks of the subproject were explained and their opinions, suggestions and concerns regarding the subproject activities were asked. In addition to not detecting any negative opinions, the transportation channels to be used in case of any complaints were provided. The ETL will pass through the parcel allocated for the subproject and be connected to the existing pole. There are no sensitive structures such as schools or health facilities on the access route and ETL route. Figure 2. Subproject AoI |
| Description of geographic and physical characteristics as appropriate. | The planned solar power plant will be located in Yenice Neighborhood, Gercüş district of Batman province, which is a rural inland area that benefits from high solar radiation and is a suitable area for solar energy production. Geographically, the region experiences a continental climate with long, sunny periods, particularly in summer, enhancing the efficiency of solar panels. The physical environment is predominantly rural, with agricultural and pasture lands surrounding the site. Additionally, the existing infrastructure, including road connections and electrical networks, supports renewable energy projects, facilitating the integration of solar power into the local grid. According to AFAD Türkiye Earthquake Map; PGA value of the sub-project area was determined as 0.106 g (see Figure 3). This value expresses the maximum acceleration that the ground can be exposed to during a possible earthquake in a certain region. In terms of structural design and engineering studies, this acceleration value should be taken into consideration and necessary resistance measures should be taken and compliance with the relevant regulations should be ensured. Figure 3. Sub-project Seismicity Map (https://tdth.afad.gov.tr/TDTH/) |
| Description of biological characteristics as appropriate. | Although SPP projects generally have low environmental impact, detailed ecological assessments are made during site selection in order not to harm biodiversity. Literature and field studies were carried out by ÇA Engineering Agricultural Engineer Mehmet Sümer on 09.02.2025 to determine the flora and fauna species present or likely to be present within the subproject area of influence. 1. Flora Literature and field studies were conducted for the determination of flora and fauna species located or likely to be located within the subproject area of influence. Flora species found in the subprojectarea of influence were identified as Eryngium campestre and Silybum marianum. Within the scope of the studies conducted in the subproject area, no endangered or endemic plant species were encountered in the subproject area of influence. In this context, the books “Flora of Türkiye and East Aagean Island (1965- 1988)” and “Red Data Book of Turkish Plants” prepared by P. DAVİS were used. In addition, the databases prepared by TUBITAK, “http://bioces.tubitak.gov.tr” and Turkish Plants Data Service – TUBITAK: “http://wwweski.tubitak.gov.tr/tubives/” were scanned and the literature was supported to check whether there were any endangered species. There are no rare, endangered or protected plant species in the subproject area of influence according to Annex 1 of the Bern Convention. 2. Fauna The fauna list in the immediate vicinity of the subproject area, based on fieldwork and literature review, is given below. In literature studies; Mustafa Kuru’s ‘Vertebrate Animals’, A. Demirsoy’s ‘Türkiye Vertebrates – Mammals, Amphibians’, İbrahim Baran’s ‘Türkiye’s Amphibians and Reptiles’, İ. Kiziroğlu’s (2008) ‘Türkiye Birds Red List’ (Species List in Red Data Book) were used. Wildlife species protected under the Bern Convention, as well as other species, are not affected by activities such as hunting, deliberate killing, captivity, or the destruction of their eggs. The decisions of the Central Hunting Commission of the Ministry of Agriculture and Forestry for 2024-2025 and the provisions of the Bern Convention will be complied with in the activity in question. Furthermore, species listed as threatened or near-threatened according to the IUCN Red List will be taken into consideration, and appropriate mitigation measures will be adopted to avoid adverse impacts. Any species listed under CITES Appendices will be strictly protected from any form of capture, trade, or exploitation in accordance with international regulations. As a result of the flora and fauna surveys conducted in the subproject area and based on consultations with local people, mammalian species including the wild boar red fox (Vulpes vulpes), wild boar (Sus scrofa), rabbit (Lepus europaeus) have been identified in the subproject area. As bird species, it was determined that species such as hawk (Buteo buteo), eagle (Aquila chrysaetos), partridge (Alectoris chukar) and quail (Coturnix coturnix) were observed. Additionally, reptile and amphibian species such as slender snake (Platyceps najadum), black snake (Dolichophis jugularis), striped lizzard (Ophisops elegans), green lizard (Lacerta viridis), Anatolian rock lizard (Anatololacerta anatolica), Spiny frog (Stellagama stellio) were observed. In this context, no species that are definitely protected under the BERN Convention were encountered. Species that are allowed to be hunted for certain periods by the decision of the Central Hunting Commission were identified, but these species are not within the subproject area but are in the immediate vicinity. It is not expected that the activities to be carried out within the scope of the filling activity subject to the subproject will have a negative impact on these species. However, within the scope of the subproject, necessary measures will be taken for the protection of wildlife in accordance with the Land Hunting Law No. 4915 and the decisions of the Central Hunting Commission held every year. In the subproject area; national parks, nature parks, nature monuments, nature conservation areas, wildlife conservation areas, wild animal breeding areas, cultural assets, natural assets, protected areas and protection areas, special environmental protection zones, biogenetic reserve area, biosphere reserve special protection areas, afforested areas, potential erosion and afforestation areas, protection areas related to drinking and utility water resources, densely populated areas, historical, cultural, archaeological and similar areas of importance, tourism regions and other protected areas; were not encountered in the database used as a source and other researches. There is no nationally protected and internationally recognized high biodiversity value areas in the subproject area of influence. Additionally, there are no World Heritage Natural Protected Areas, Biosphere Reserves, Ramsar Wetlands of International Importance, Important Biodiversity Areas or Important Bird Areas. Furthermore, field surveys and desktop studies have confirmed the absence of any critical habitats, endemic or endangered species, or ecologically sensitive zones within the subproject area of influence. |
| Description of geological and hydrographic characteristics as appropriate. | The selected subproject site’s geological and hydrographic characteristics play a crucial role in assessing the feasibility and sustainability of the solar power plant. The geological conditions, including soil composition, bedrock stability, and seismic activity, influence the foundation design and construction process. Additionally, hydrographic features such as surface water bodies, groundwater levels, and drainage patterns are evaluated to ensure minimal impact on local water resources. This section provides an overview of the site's geological and hydrographic attributes, highlighting any relevant environmental considerations for the subproject development. - Geology The area where the sub-project site is located has geologically characteristic lithologies of the Southeastern Anatolia Region. The region is generally composed of sedimentary rocks such as limestone, marl and claystone belonging to the Mardin Group. In addition, Batman and its surroundings contain widely distributed carbonate rocks such as Midyat Limestones, and these units contribute to the groundwater potential with their high permeability properties. The topographic structure in the region is rugged and includes valleys and plateau surfaces that have been geomorphologically exposed to river erosion. Structurally, fractures and fault lines can be observed in places, indicating that the region has been exposed to tectonic movements. The subproject area geology map is given in Figure 4. Figure 4. Subproject Site Geology Map (Ministry of Agriculture and Forestry National Water Information System) - Hydrographic characteristics According to the National Water Information System; there is no underground water source in the sub-project area. Considering the seasonal flow changes of the dry streambed, there is no risk of flooding, especially during rainy periods. However, there is a dry streambed 200 meters north of the sub-project site. As a result of interviews with local people during the field visit, it was determined that seasonal flows were observed in the dry streambed. The map showing the closest water source to the sub-project area is shared in Figure 5. Figure 5. Subproject Site Water Resources Map (Ministry of Agriculture and Forestry National Water Information System) Although not identified in the National Water Information System, a dry streambed was observed approximately 200 meters north of the sub-project site during the field survey. This streambed plays a minor hydrological role by channeling seasonal surface runoff, such as melting snow and rainwater. However, due to its distance from the sub-project site and the absence of direct interaction with project activities, it has not been included within AoI. Figure 6. Dry stream bed |
| Description of socio-economic characteristics as appropriate. | The primary economic activities in Yenice Neighborhood, the nearest settlement to the project site, are agriculture and animal husbandry. The renovation and capacity expansion activities planned within the scope of the project are expected to generate temporary employment opportunities in the construction phase. During this period, priority will be given to supporting the local economy by procuring materials locally and sourcing various goods and services from the surrounding area. Furthermore, no dedicated worker accommodation or campsite will be established during the construction phase. As part of the project's commitment to social and economic sustainability, efforts will be made to recruit personnel primarily from the local population whenever feasible. |
| If relevant, provide information about the affected settlements. | Closest settlement: - Yenice neighborhood (with 304 population, according to TurkStat, 2024) Closest structure(s) to the Subproject site: - Country House located in Yenice village at 200 m north of the subproject site. |
| Locations and distance to nearest sensitive receptors such as health care units, schools? | The nearest sensitive receptor is a garden house located approximately 200 meters from the subproject site. There is also school etc 1000 meters away. |
| Infrastructure services to be used during the life cycle of the subproject (sewage, electricity, water network, etc.) | Due to the subproject location, electricity, water will be supplied from the current infrastructure networks. No need to construct/renew infrastructure services due to subproject activities. |
| 1.c) E&S Requirements applicable to the Subproject |  |
| The subproject will be implemented in line with requirements of applicable national legislation and international agreements and conventions to which Türkiye is a party of. The following international standards will also be followed as applicable: - WB Environmental and Social Framework (ESF, 2018) and the Environmental and Social Standards (ESSs) forming part of the ESF - Good International Industry Practices (GIIPs) including but not limited to WB Group (WBG) General and Industry Sector Environmental, Health and Safety Guidelines (EHSGs) - International Finance Corporation (IFC) ESMS Implementation Handbook - ILBANK Environmental and Social Management System (ESMS) In cases where the requirements of the ILBANK ESMS or national legislation differ from those of the WB ESSs or the levels and measures presented outline in the relevant WBG EHS guidelines, the more stringent standard will apply. |  |

# Part 2: Implementation Arrangements

2.b) Implementation Responsibility and Resources

The sub-borrower shall implement and cause the contractor to be adopt and implement this ESMP Checklist satisfactory to ILBANK throughout the sub-financing agreement life cycle. The sub-borrower is responsible for ensuring that adequate financial and human resources are allocated for the effective implementation of this ESMP Checklist. Roles and Responsibilities are provided in Appendix-10.

2.c) Organizational Capacity

Sub-borrower: The sub-borrower shall establish an organizational structure (Project Implementation Unit – PIU) with qualified staff and resources to the satisfaction of ILBANK and maintain it by ensuring that there is qualified staff assigned and serving on the duty throughout the sub-financing agreement life cycle. The sub-borrower assigns the following personnel to support management and monitoring of subproject E&S risks and impacts and ensure full compliance with the requirements of this ESMP Checklist: - Environmental Focal Point:, Administrative staff, 11 - Social Focal Point ((who will also act as the Grievance Mechanism (GM) Focal Point): A staff member from the Administrative Unit (Seniority Level: 7) Occupational Health and Safety (OHS) Specialist: Sabahattin Ertuğrul, Administrative staff, 8

Contractors: The sub-borrower shall obligate awarded contractors to establish and maintain throughout the contract duration an organizational structure with qualified staff and resources. This shall be achieved through assigning the following personnel under the contractor’s organization prior to commencement of works: - One (1) Environmental Specialist: Please insert name-surname, position/title, length of professional experience - One (1) Social Specialist: Please insert name-surname, position/title, length of professional experience - One (1) Occupational Health and Safety (OHS) Specialist: Please insert name-surname, position/title, length of professional experience, expertise class The sub-borrower shall in writing notify ILBANK of the assigned contractor personnel prior to commencement of works.

2.d) Grievance Mechanism (GM)

The grievance mechanism of the subproject shall be implemented in line with the ILBANK’s Grievance Mechanism which is available at https://www.ilbank.gov.tr/userfiles/files/Grievance_Mechanism.pdf The sub-borrower shall require the contractors to prepare notification board/sign and post them at or around the worksite prior to commencement of works. The board/sign will include contact details for public to raise their subproject-related grievances and feedback . The grievances shall be recorded by the sub-borrower, construction supervision consultant and the contractor, and submitted to ILBANK weekly. Sensitive complaints will be reported to WB within 48 hours following the grievance received by ILBANK.

2.e) Monitoring and Reporting

The sub-borrower shall promptly notify ILBANK of any incident or accident related to the subproject which has, or is likely to have, a significant adverse effect on the environment, the affected communities, the public or workers, including, inter alia, cases of sexual exploitation and abuse (SEA), sexual harassment (SH), and accidents that result in death, serious or multiple injury. This notification shall be done by using ILBANK’s E&S Incident Notification Form template (see Appendix-8). Completed E&S Incident Notification Form shall be submitted to ILBANK by the sub-borrower within 48 hours of the incident or accident (contractor shall notify the sub-borrower within 24 hours of the incident or accident). The periodic E&S monitoring reporting requirements for the subproject is as follows: - Construction contractor will prepare monthly E&S monitoring reports (ESMRs) and submit to supervision consultant (“müşavir”). - During the construction phase, the sub-borrower, with support from supervision consultant, will prepare quarterly ESMRs and submit to ILBANK. - During the operation phase (throughout sub-financing agreement lifecycle, until the completion of repayment period), the sub-borrower will prepare annual ESMRs and submit to ILBANK. ILBANK will provide the sub-borrower with the required template for the periodic ESMRs. The Roles and Responsibilities are provided in Appendix-10.

2.f) Public Consultation

Within the scope of the meeting, the environmental expert from the consulting firm presented the environmental and social risks identified in the Environmental and Social Management Plan (ESMP). The expert also described the measures planned to mitigate these risks, the potential impact of the geographical location and climatic conditions, and the assessments conducted regarding possible natural disasters in the project area. The session concluded with a question-and-answer segment, during which participants were given the opportunity to share their opinions, suggestions, and concerns. The meeting lasted approximately 45 minutes. A record of the meeting and participant list is provided in Appendix-12.

# Part 3: ESMP Matrix: Risk and Impacts, Mitigation and Monitoring

## 3.a) Construction ESMP Matrix

| No. | Risks and Impacts | Receptor(s) | Proposed Mitigation Measure | Responsible Parties |
| --- | --- | --- | --- | --- |
| Labor and Working Conditions |  |  |  |  |  |
|  | Working Conditions | Construction workforce Employees | - Conduct daily toolbox talks covering the OHS Plan and labor conditions and record it to be displayed when requested. - Develop and implement a subproject-specific simplified Labor Management Procedure (SLMP, see Appendix-7.) to ensure compliance in recruiting and managing all employees. - Enforce strict prohibition of child labor, forced labor, and unregistered labor as per SLMP requirements. - Provide employees with clear, documented information on their labor rights, including working hours, wages, overtime, compensation, and benefits at the start of employment and whenever material changes occur. - Implement and maintain an accessible Grievance Mechanism for workers. Inform all workers at recruitment. | Kayapınar Municipality Supervision Consultant Contractor |
|  | General OHS risks | Construction workforce | - Develop a comprehensive risk assessment document and Permit to Work Procedure for sub-project, addressing specific risks and defining mitigation measures and follow the operating procedure. - For non-routine work, a risk assessment will be conducted before starting. - A work permit procedure will be implemented. - Ensure that all employees, including subcontractors, receive necessary OHS training covering identified risks. - Prepare sub-project management plans, including Safe Work Procedures and an Emergency Response Plan. - Emergency scenarios should be determined for all possible risks and every employee should receive training on these scenarios. - Enforce safety procedures and provide appropriate PPE to all employees. - Incorporate job-specific safety procedures and requirements in OHS training programs. - Prepare machine and operation specific “Safe Working Procedures” for all safety critic equipment and machinery and notify all workforce by signature. - Serious safety issues that may arise with primary suppliers and primary supply workers will be managed as described in the Occupational Health and Safety Sub-Management Plan, which will cover primary supply workers to the extent necessary. - Written contracts will be provided to subcontractors, setting out detailed job descriptions, rights and obligations, and a Code of Conduct. - In case of OHS accidents resulting in loss of life, 24 hours) and follow up by filling in the Environmental and Social Reporting Template (ESRT) forms in accordance with the instructions of ILBANK. This process shall also include root cause analysis and corrective action plan. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Physical Hazards: Lifting Operations OHS Risks | Construction workforce | - Ensure that lifting area will be enclosed with fence to prevent access to the lifting area during lifting work. - Ensure that warning signs will be installed for lifting activities - Ensure that safety procedures will be used for lifting operations. - Ensure that lifting work will be carried out by well trained, qualified, and certified lifting team and with proper communication means and flag man. - Ensure that workers will be provided with all necessary PPE and safety materials. - Ensure all equipment used for lifting operations including slings, chains and hooks are checked technically and records are kept according to local safety legislation. - Ensure that tools are selected and designed that reduce force requirements and holding times and improve postures. - Ensure that user-adjustable workstations are provided. - Ensure that rest and stretch breaks are incorporated into work processes and job rotation is in place. - Ensure that quality control and maintenance programs are in place to reduce unnecessary forces and effort, and personnel are trained in proper manual handling techniques. - Ensure that additional special circumstances, such as left-handed people, are considered. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Physical Hazards: Rotating and Moving Equipment | Construction workforce | - Design machines to eliminate trap hazards and ensure that extremities are kept out of harm’s way under normal operating conditions; i.e. availability of emergency stops dedicated to the machine and placed in strategic locations; - If a machine or equipment has an exposed moving part or an exposed pinch point that could endanger the safety of any worker, ensure that the machine or equipment is equipped with and protected by a guard or other device that prevents access to the moving part or pinch point. Guards should be designed and installed in conformance with appropriate machine safety standards; - Ensure that machinery with exposed or protected moving parts or in which energy can be stored (e.g. compressed air, electrical components) is turned-off, disconnected, isolated and de-energized (Locked Out and Tagged Out) during service or maintenance; - Where possible, ensure that equipment is designed and installed to enable routine servicing, such as lubrication, to be carried out without removing guarding devices or mechanisms. - if extension cables are to be used, they shall be selected according to the current to be carried and the length of the cable. Extension cables are for temporary use. Outdoor extension cables will be used. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Physical Hazards: Electrical Hazards | Construction workforce | - Ensure that all energized electrical devices and lines are marked with warning signs; - Ensure that the devices are locked (de-charging and leaving open with a controlled locking device) and labeled (warning sign placed on the lock) during service or maintenance; - Ensure that all electrical cords, cables, and hand power tools are checked for frayed or exposed cords. Also, ensure that the manufacturer's recommendations for the maximum permitted operating voltage of portable hand tools are followed; - Insulating mats should be placed under electrical panels, including portable panels. - Ensure that portable electrical appliances are “portable electrical appliance tested (PAT)” - Ensure that all electrical equipment used in environments that are or may be wet is double insulated/grounded; use equipment with ground fault interrupter (GFI) protected circuits; - Residual Current Devices (RCDs) or Ground Fault Interrupters (GFIs) shall be installed in all electrical panels, including temporary and permanent ones, to protect against electrocution risks. The functionality of these devices shall be checked regularly during both the construction and operational phases to ensure continuous safety. - Ensure that power cords and extension cords are protected against damage from traffic by shielding or suspending above traffic areas; - If extension cords are to be used, they must be selected based on the current they are required to carry and their total length. Extension cords are intended for temporary use only, and outdoor-rated extension cords must be used in external environments to ensure safety and durability. - Ensure that high-voltage equipment ('electrical hazard') and service rooms where access is controlled or prohibited are properly labeled; - Ensure that "No Approach" zones are established around or under high voltage lines; - Ensure that construction vehicles or other vehicles with rubber tires that come into direct contact with or arc across high-voltage cables are taken out of service for 48 hours; - Ensure that all buried electrical cables are thoroughly identified and marked prior to any excavation work. - Ensure that special training programs are organized for employees on electrical hazards and safety precautions. - Ensure that rapid response teams and emergency plans are established for electrical accidents. - Ensure that regular electrical safety inspections are conducted in the project area. - Ensure that periodic inspections are conducted to ensure that employees use appropriate personal protective equipment (PPE). - . Fire extinguishers containing CO2 should be provided for possible electrical fires. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Physical Hazards: Welding and Hot Works | Construction workforce | - Ensure that appropriate eye protection, such as welder's goggles and/or a full-face eye shield, is provided for all personnel involved in or assisting with welding operations; - If welding or hot cutting is performed outside of established welding work stations, ensure that special hot work and fire prevention precautions and Standard Operating Procedures (SOPs) are in place, including "Hot Work Permits, stand-by fire extinguishers, fire blanket, stand-by fire watch and maintaining fire watch for up to one hour after welding or hot cutting is finished"; - Ensure that areas where welding or hot work is performed are cleared of flammable materials (e.g. fuel, solvent, spark-ignitable materials) and should be checked regularly. - Ensure that all employees are trained and informed about welding operations and the safe management of hot work. - Ensure that welding work is only carried out by employees who have the appropriate professional qualification (aluminum, steel, resistance etc.) | Kayapınar Municipality Supervision Consultant Contractor |
|  | Fire Safety Prevention Measures and Emergency Response | Construction workforce Flora and fauna Soil, water resources | - Ensure all employees are trained for their responsibility to report dangers and fire fighting measures - Ensure that all flammable and hazardous materials are stored in designated, secure areas away from ignition sources. - Ensure firefighting systems and equipment are available - Ensure fire and emergency drills are conducted regularly. - Designate trained fire wardens for each area to lead evacuations and coordinate with emergency responders. - Keep an up-to-date list of emergency contacts, including local fire departments and hospitals, for quick access in case of fire. - Ensure the number of trained first-aiders is in accordance with workplace hazards class as specified in the First Aid Regulation. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Physical Hazards: Ergonomics, Repetitive Motion, Manual Handling Lifting | Construction workforce | - Establish clear weight limits for manual handling tasks and label heavy loads accordingly; - Ensure that mechanical assists are used to eliminate or reduce the effort required to lift materials, hold tools and work objects, and that more than one person is lifting if weights exceed thresholds; - Ensure that tools are selected and designed that reduce force requirements and holding times and improve postures; - Ensure that user-adjustable workstations are provided; - Ensure that rest and stretch breaks are incorporated into work processes and job rotation is in place; - Ensure quality control and maintenance programs are in place that reduce unnecessary forces and effort; - Ensure that additional special circumstances, such as left-handed people, are considered. - Whether a new employee can carry heavy loads should be determined during a health check by the workplace doctor. Make sure that these jobs are performed by people who are approved. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Physical Hazards: Industrial Vehicle Driving and Site Traffic | Construction workforce | - Ensure that industrial vehicle operators are trained in the safe use of specialized vehicles such as forklifts, including safe loading/unloading, load limits; - Make sure that they have a certificate of competence (driver's license, operator's certificate, etc.) from authorized institutions and organizations according to the type of work machine. - Make sure drivers undergo medical supervision; - Ensure that moving equipment with restricted rear visibility is equipped with audible back-up alarms; - Ensure that rights of way, site speed limits, vehicle inspection requirements, operating rules and procedures (e.g. prohibiting operation of forklifts with forks down), and control of traffic patterns or direction are established; - Ensure that deliveries and movement of private vehicles are restricted to defined routes and areas, with 'one-way' movement preferred where appropriate. - . While the work equipment is in motion, it is prevented that anyone approaches the blind spots of the equipment. In cases where this cannot be achieved, object detection systems or warning systems are used. If this cannot be achieved either, a signal is used, coordination is ensured with the people around, or no one is allowed to enter the work area. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Physical Hazards: Chemical Hazards | Construction workforce Flora and fauna Soil, water resources | - Ensure that the hazardous substance is replaced with a less hazardous substitute; - Ensure that engineering and administrative control measures are in place to prevent or minimize the release of hazardous substances into the working environment, keeping the exposure level below internationally established or recognized limits; - Ensure that the number of workers exposed or likely to be exposed is minimal; - Ensure that chemical hazards are communicated to workers through labeling and marking according to nationally and internationally recognized requirements and standards, including International Chemical Safety Cards (ICSC), Safety Data Sheet or equivalent. Any means of written communication should be in an easily understood language and be readily available to exposed workers and first-aid personnel; - Ensure that employees are trained in the use of available information (such as Safety Data Sheet), safe working practices and proper use of PPE. - Ensure workers have access to suitable personal protective equipment (PPE), such as gloves, respirators, goggles, and protective clothing, based on the specific chemical hazards. - Store hazardous substances in designated areas with appropriate ventilation, labeling, and secure containment to prevent accidental exposure or spills. - Provide chemical overflow pallets and store chemical containers by placing them on them. - Develop and implement a spill response plan, as part of the Emergency Response Plan, including containment measures, chemical spill/leak response drills, cleanup procedures, hazardous substance disposal, and emergency contact information. - Dispose of chemical waste according to regulations to prevent environmental contamination and worker exposure. - Regularly inspect and maintain chemical handling equipment, storage areas, and PPE to prevent leaks or accidental releases. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Gender-Based Violence (GBV); Sexual Exploitation and Abuse/Sexual Harassment (SEA/SH) on Employees; Gender Inequality | Construction workforce | - Provide GBV and SEA/SH awareness sessions for the management teams of the construction contractor and consultants to promote understanding and accountability. - Conduct regular awareness meetings with workers to educate them on GBV and SEA/SH issues and the importance of respectful workplace conduct. - Ensure all workers receive training on recognizing, preventing, and responding to GBV and SEA/SH incidents. - Require all workers to review, sign, and adhere to a Code of Conduct that explicitly addresses unacceptable behaviors related to GBV and SEA/SH. - Implement a confidential and accessible grievance mechanism specifically designed to capture and address GBV and SEA/SH-related complaints in a timely manner. | Kayapınar Municipality Supervision Consultant Contractor |
| Resource Efficiency and Pollution Prevention and Management |  |  |  |  |  |
|  | Waste Management - General | Communities Construction workforce Flora and fauna Soil, water resources | - Separate wastes at the source into waste categories determined in Waste Management Regulation, establish temporary waste storage area - Place labeled bins for each type of waste at strategic locations on-site to ensure correct disposal by workers. - Implement practices to reduce waste generation by optimizing material use and reusing materials where possible. - Contract with local recycling facilities to ensure that recyclable materials (e.g., metals, paper, plastic) are properly processed. - Store waste in designated, secured areas to prevent littering, leaching, and environmental contamination. - Use leak-proof containers for hazardous or liquid waste and ensure they are adequately labeled. - Contract with licensed waste disposal companies to handle non-recyclable and hazardous wastes in accordance with Waste Management Regulation. - Track and document the disposal process to ensure compliance and accountability. - Conduct regular awareness sessions and training for workers on waste reduction techniques, proper disposal practices, and the importance of waste management. - Regularly monitor waste management practices, conduct site inspections, and assess waste volumes to identify areas for improvement. - Establish a reporting system to document waste types, quantities, and disposal methods. - Develop a comprehensive waste management plan that includes waste reduction targets, disposal methods, monitoring schedules, and assigned responsibilities for effective waste management throughout the project. - Use containment systems for waste that poses spill risks, and keep spill kits accessible. Train staff on immediate spill response actions to prevent soil and water contamination. - Conduct maintenance tasks, such as oil changes and battery replacements, off-site; | Kayapınar Municipality Supervision Consultant Contractor |
|  | Waste Management -Electronic Waste Disposal | Communities Construction workforce Soil and water resources | - Contract with recycling facilities and/or manufacturers to ensure proper disposal or recycling of obsolete equipment; - Agreements will be set with e-waste recycling facilities to ensure responsible disposal of electronic waste from solar panels, inverters, batteries, etc. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Wastewater Management | Flora and fauna Soil, water resources | - Portable toilet will be placed on a level, solid surface. - Toilet will be located away from food and beverage preparation and rest areas. - Toilet will be secured against wind or tipping. - The interior of the toilet will be cleaned and disinfected regularly. - Toilet will be emptied periodically with a vacuum truck, depending on occupancy. - Soap, disinfectant, and paper towels will be placed inside. - Deodorizing chemicals will be added regularly. - Ventilation openings in the toilets will be kept clear. - The area surrounding the toilets will be made safe with non-slip material. - Employees will be trained on the proper and hygienic use of portable toilet. - Cleaning and maintenance procedures will be recorded. . | Kayapınar Municipality Supervision Consultant Contractor |
|  | Soil and Groundwater Contamination | Communities Construction workforce Flora and fauna Soil and water resources | - Contain and clean up any oil, chemical, lubricant, or fuel spill immediately to prevent environmental contamination. - Implement spill prevention and response measures. Maintain spill containment and clean-up kits on-site. Ensure all spills are contained, cleaned, and disposed of by licensed waste management companies. - Conduct routine servicing of construction vehicles and equipment at designated off-site locations to minimize the risk of leaks or spills. - Perform refuelling in designated areas following strict protocols to prevent accidental spills. - Collect and store waste oil securely for recycling or dispose of it through licensed waste vendors to ensure safe handling. - Provide adequate sanitary facilities, including toilets and showers, for the construction workforce. Ensure prompt repairs and maintenance in the event of any leaks or spills to maintain hygiene and safety standards. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Dust and Gaseous Emissions | Communities Construction workforce Flora and fauna Ambient air quality | - Apply water spraying to suppress dust when dusting occurs on roads and construction area. Use water tankers to supply water for this purpose. - Inform communities/residential areas nearby about the schedule and nature of construction activities as part of the Stakeholder Engagement Plan (SEP). - Carry out loading and unloading of trucks carefully to prevent materials from dispersing or scattering. - Cover transport trucks with tarpaulins on public roads when arriving at or leaving the site to minimize dust. Clean truck tires before leaving the site to prevent mud and debris from spreading onto public roads. - Enforce a speed limit for trucks to reduce dust and improve site safety. - Use modern equipment and vehicles that meet relevant emission standards. Regularly inspect and maintain exhaust systems to ensure emission levels remain within safe limits. - Implement good site practices by using low-emission construction equipment and vehicles. Utilize cleaner fuels and technologies to reduce dust and other airborne pollutants. - Implement a grievance mechanism to address community concerns. Halt work in case of grievances until corrective measures are in place. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Environmental Noise | Communities Construction workforce | - Prohibit the operation of construction machinery at night to minimize noise disturbances. - Inform communities/residential areas nearby about the timing and nature of construction activities as part of the Stakeholder Engagement Plan (SEP). - Ensure that machinery and equipment used during land preparation and construction are distributed evenly throughout the site rather than concentrated in one location. - Choose construction machinery and equipment with low noise emissions to minimize noise impact on the surrounding area. - Use noise barriers or enclosures for loud equipment. - Conduct regular and periodic maintenance of construction machinery and equipment, including daily checks before each shift, to ensure optimal performance and reduce noise levels. - Ensure all vehicles used for transportation comply with the speed limits to minimize noise and enhance safety. - Establish a grievance mechanism to receive and address complaints related to noise and other nuisances from the community. - Halt construction activities in response to grievances until appropriate preventive measures are implemented to address the issues raised. - In case of any environmental noise complaints, measurements will be conducted by accredited laboratory to determine the environmental noise level caused by construction work and if it is over the limits, additional measures such as barriers, arrangement of working hours, etc. will be taken. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Hazardous Substances Management | Construction workforce Communities Flora and fauna Soil and water resources | - Maintain a comprehensive record of the types, quantities, and properties of hazardous materials to be stored on-site. - Establish a designated storage area specifically equipped for the safe storage of hazardous and toxic materials. - Ensure all storage containers are clearly labelled with appropriate hazard warnings, safety information, and emergency contact details to facilitate proper handling and identification. All chemicals will be managed in accordance with their Material Safety Data Sheets (MSDS). - Utilize suitable containers, tanks, and bunding systems to contain hazardous materials and prevent spills, leaks, or releases. Implement secondary containment measures, such as berms, dikes, or containment basins, to capture any accidental releases. - Ensure adequate ventilation and venting systems are in place within storage areas to prevent the accumulation of hazardous vapours or gases. - Identify and safely remove hazardous materials, including lead-containing components from solar panels and electronic waste from inverters, following proper disposal protocols. - Implement appropriate containment and handling procedures to minimize the risk of spills or releases of hazardous substances during storage and handling. - Arrange for the proper disposal or recycling of hazardous materials through licensed facilities to ensure safe and compliant waste management. | Kayapınar Municipality Supervision Consultant Contractor |
| Community Health and Safety |  |  |  |  |  |
|  | Increased traffic | Communities | - Coordinate traffic management to regulate construction vehicle movement. - Schedule construction activities during off-peak hours to minimize traffic congestion. - Ensure coordination and develop infrastructure upgrades or expansions in advance of the sub-project, including improvements to roads, utilities, and telecommunications if necessary. - Use flagmen and signage to direct traffic safely around construction area. - Provide regular updates to the community about construction schedules and traffic impacts. - Ensure all construction vehicles comply with speed limits specified in the regulations and are maintained to minimize emissions and noise. - Limit vehicle speed on unpaved roads to 30 km/h. - Conduct safety training for construction workers on road safety protocols and provide road safety training for all drivers. - Use safe traffic control measures, including road warning signs, speed bumps, and flag persons as necessary. - Monitor traffic conditions and adjust operations as necessary to ensure safety. - Repair any damage to the roads promptly. - Establish a grievance mechanism for community members to report traffic concerns. - Prepare an emergency response plan and protocols to address potential infrastructure failures, accidents, or natural disasters during construction. - Place warning signs, speed bumps and signaling systems on roads passing in front of the school. - Restrict construction site vehicles from passing through the area during school entrance and exit hours or determine alternative routes. - Before construction work that may cause temporary disturbance, the public and nearby institutions and organizations, hospitals and schools will be informed. - Assign direction officers to ensure safe passage of service vehicles and pedestrians - Vehicles carrying project materials would not park outside the subproject area. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Risks related with Gender Based Violence (GBV) Sexual Exploitation Abuse / Sexual Harassment (SEA/SH) | Communities | - Deliver ethical rules and public communication training to all employees to prevent gender-based violence (GBV), harassment, and abuse in the workplace. - Require all workers to sign and adhere to a code of conduct that promotes respectful behaviour. - Conduct regular awareness-raising sessions on-site focused on GBV prevention and other relevant social issues. - Establish a grievance mechanism to receive and address complaints related to GBV and workplace misconduct. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Local Economy, Livelihood Sources and Employment | Communities | - Prioritize local hiring for unskilled, semi-skilled, and skilled positions within the scope of the sub-project. - Regularly engage with local communities and maintain a grievance mechanism to address community concerns and feedback. | Kayapınar Municipality Supervision Consultant Contractor |
|  | Impacts on Vulnerable and Disadvantaged Individuals and Groups | Communities | - Develop a recruitment policy that includes non-discriminatory hiring practices, tailored training programs for vulnerable groups, and support services such as transportation and childcare to facilitate workforce participation. | Kayapınar Municipality Supervision Consultant Contractor |
| Land Acquisition, Restrictions on Land Use and Involuntary Resettlement |  |  |  |  |  |
|  | Impacts on Local Communities using the Site and Its Environs for Grazing, Agriculture or Beekeeping | Communities | - Ensure availability of grievance mechanism for stakeholders affected by land use. - It will be ensured that construction activities do not restrict/obstruct the social and economic life of the local community. - Private and public lands outside the project work area will not be entered and all measures will be taken to prevent this. | Kayapınar Municipality Supervision Consultant Contractor |
| Biodiversity Conservation and Sustainable Management of Living Natural Resources |  |  |  |  |  |
|  | Disturbance on Biodiversity | Flora and fauna | - Engage qualified biodiversity experts to conduct pre-construction surveys to identify presence and distribution of flora and fauna on the subproject site, if any, with a focus on impact on habitats such as nesting or burrowing sites, to avoid disturbance or destruction during construction activities. - Implement a gradual construction approach to allow fauna species time to escape or provide for their relocation to suitable habitats. - Schedule construction activities during periods of low wildlife activity, avoiding nesting seasons for birds and hibernation periods for mammals. - Minimize vegetation removal by conducting thorough surveys to avoid unnecessary clearing. - Restore natural vegetation upon completion of construction activities to enable species to re-inhabit surrounding areas. - Topsoil, carefully stripped prior to subproject activities, will be stored separately and maintained under appropriate conditions. This soil will be reused during landscaping and site restoration works. - Install exclusion fencing to prevent animals from entering construction zones, using wildlife-friendly designs that allow small animals to pass through safely. - Install barriers around known burrows or nesting sites to protect them from disruption during construction, using temporary or permanent solutions as necessary. - Clearly separate subproject construction sites and access roads from other areas with appropriate signage and fencing, limiting personnel and vehicle access to these areas. - Reduce habitat degradation by keeping vehicles on designated access roads and minimizing pedestrian traffic in intact areas. | Kayapınar Municipality Supervision Consultant Contractor |
| Cultural Heritage |  |  |  |  |  |
|  | Impacts on Cultural Heritage | Cultural heritage | - Develop and implement the Chance Finds Procedure (see Appendix-9) to ensure timely identification and appropriate management of any chance findings during sub-project implementation. - Include the Chance Finds Procedure as part of toolbox training sessions during construction to raise awareness among workers. - Stop construction work immediately if any chance finds are encountered. - Inform the relevant Preservation Board or Museum Directorate immediately, and ensure the security of the area by the contractor. Construction work will not resume until official notification is received. | Kayapınar Municipality Supervision Consultant Contractor |
| Stakeholder Engagement and Information Disclosure |  |  |  |  |  |
|  | Insufficient Stakeholder Engagement Activities and Public Consultation. | Communities Construction workforce | - Create channels for interaction and communication with local communities, ensuring that engagement activities are scheduled at convenient times. - Conduct regular consultations with relevant authorities and local communities to discuss project management and gather feedback. - All channels of reaching out to the local people will be used to increase participation. Bulk SMS, WhatsApp messages, social media channels, posters and brochures will be prepared and delivered to the local people, especially the brochures will be hung in mukhtar offices, mosques, tea houses and coffee houses. In addition, a section will be created for the sub-project on the Kayapınar Municipality website. All information about the sub-project will be shared here. The support they need will be provided to vulnerable and disadvantaged groups who may have difficulty in participation. | Kayapınar Municipality Supervision Consultant Contractor |

## 3.b) Operation ESMP Matrix

| Ref. | Impact Description | Sensitive Receptor(s) | Management/ Mitigation Measure | Responsibility for Implementation of Mitigation Measure |
| --- | --- | --- | --- | --- |
| Labor and Working Conditions |  |  |  |  |  |
|  | Improper Working Conditions | Employees | - Conduct daily/weekly toolbox talks covering the OHS Plan and labor conditions and record it to be displayed when requested.. - Apply the SLMP to ensure compliance in recruiting and managing all employees. - Enforce strict prohibition of child labor, forced labor, and unregistered labor as per SLMP requirements. - Provide employees with clear, documented information on their labor rights, including working hours, wages, overtime, compensation, and benefits at the start of employment and whenever material changes occur. - Implement and maintain an accessible Grievance Mechanism for workers. Inform all workers at recruitment. | Kayapınar Municipality |
|  | General OHS risks | Employees | - Develop a comprehensive risk assessment document and Permit to Work Procedure for sub-project, addressing specific risks and defining mitigation measures and follow the operating procedure. - For non-routine work, a risk assessment will be conducted before starting. - A work permit procedure will be implemented. - Ensure that all employees, including subcontractors, receive necessary OHS training covering identified risks. - Prepare sub-project management plans, including Safe Work Procedures and an Emergency Response Plan. - Emergency scenarios should be determined for all possible risks and every employee should receive training on these scenarios. - Enforce safety procedures and provide appropriate PPE to all employees. - Incorporate job-specific safety procedures and requirements in OHS training programs. | Kayapınar Municipality |
|  | Physical Hazards: Lifting Operations OHS Risks | Employees | - Ensure that lifting area will be enclosed with fence to prevent access to the lifting area during lifting work. - Ensure that warning signs will be installed for lifting activities - Ensure that safety procedures will be used for lifting operations. - Ensure that lifting work will be carried out by well trained, qualified, and certified lifting team and with proper communication means and flag man. - Ensure that workers will be provided with all necessary PPE and safety materials. - Ensure all equipment used for lifting operations including slings, chains and hooks are checked technically and records are kept according to local safety legislation. | Kayapınar Municipality |
|  | Physical Hazards: Electrical Hazards | Employees | - Ensure that all energized electrical devices and lines are marked with warning signs; - Ensure that the devices are locked (de-charging and leaving open with a controlled locking device) and labeled (warning sign placed on the lock) during service or maintenance; - A “Lockout Tagout” (LOTO) Procedure specific to the subproject should be prepared, personnel should be trained and its implementation should be supervised. - Ensure that all electrical cords, cables, and hand power tools are checked for frayed or exposed cords. Also, ensure that the manufacturer's recommendations for the maximum permitted operating voltage of portable hand tools are followed; - if extension cables are to be used, they shall be selected according to the current to be carried and the length of the cable. Extension cables are for temporary use. Outdoor extension cables will be used. - Ensure that all electrical equipment used in environments that are or may be wet is double insulated/grounded; use equipment with ground fault interrupter (GFI) protected circuits; - Damaged electrical cables should be replaced with new ones, and no taping or splicing should be done. - Insulating mats should be placed under electrical panels, including portable panels an all panels will be equipped with a residual current relay. - Ensure that portable electrical appliances are “portable electrical appliance tested (PAT)” - If extension cables are used, they will be selected according to the current they must carry and the length of the cable. - Ensure that power cords and extension cords are protected against damage from traffic by shielding or suspending above traffic areas; - Ensure that high-voltage equipment ('electrical hazard') and service rooms where access is controlled or prohibited are properly labeled; - Ensure that "No Approach" zones are established around or under high voltage lines; - Ensure that construction vehicles or other vehicles with rubber tires that come into direct contact with or arc across high-voltage cables are taken out of service for 48 hours; - Ensure that all buried electrical cables are thoroughly identified and marked prior to any excavation work. - Ensure that special training programs are organized for employees on electrical hazards and safety precautions. - Ensure that rapid response teams and emergency plans are established for electrical accidents. - Ensure that regular electrical safety inspections are conducted in the project area. - Ensure that periodic inspections are conducted to ensure that employees use appropriate personal protective equipment (PPE). | Kayapınar Municipality |
|  | Fire Safety Prevention Measures and Emergency Response | Employees Flora and fauna Soil, water resources | - Ensure all employees are trained for their responsibility to report dangers and fire fighting measures - Ensure that all flammable and hazardous materials are stored in designated, secure areas away from ignition sources. - Ensure firefighting systems and equipment are available. - Ensure fire and emergency drills are conducted regularly. - Designate trained fire wardens for each area to lead evacuations and coordinate with emergency responders. - Keep an up-to-date list of emergency contacts, including local fire departments and hospitals, for quick access in case of fire. - Ensure the number of trained first-aiders is in accordance with workplace hazards class as specified in the First Aid Regulation. | Kayapınar Municipality |
|  | Physical Hazards: Ergonomics, Repetitive Motion, Manual Handling Lifting | Employees | - Establish clear weight limits for manual handling tasks and label heavy loads accordingly; - Ensure that mechanical assists are used to eliminate or reduce the effort required to lift materials, hold tools and work objects, and that more than one person is lifting if weights exceed thresholds; - Ensure that tools are selected and designed that reduce force requirements and holding times and improve postures; - Ensure that user-adjustable workstations are provided; - Ensure that rest and stretch breaks are incorporated into work processes and job rotation is in place; - Ensure quality control and maintenance programs are in place that reduce unnecessary forces and effort; - Ensure that additional special circumstances, such as left-handed people, are considered. - . Ergonomics training should be given to personnel at regular intervals. | Kayapınar Municipality |
|  | Physical Hazards: Chemical Hazards | Employees Flora and fauna Soil, water resources | - Ensure that the hazardous substance is replaced with a less hazardous substitute; - Ensure that engineering and administrative control measures are in place to prevent or minimize the release of hazardous substances into the working environment, keeping the exposure level below internationally established or recognized limits; - Ensure that the number of workers exposed or likely to be exposed is minimal; - Ensure that chemical hazards are communicated to workers through labeling and marking according to nationally and internationally recognized requirements and standards, including International Chemical Safety Cards (ICSC), Safety Data Sheet or equivalent. Any means of written communication should be in an easily understood language and be readily available to exposed workers and first-aid personnel; - Ensure that employees are trained in the use of available information (such as Safety Data Sheet), safe working practices and proper use of PPE. - Ensure workers have access to suitable personal protective equipment (PPE), such as gloves, respirators, goggles, and protective clothing, based on the specific chemical hazards. - Store hazardous substances in designated areas with appropriate ventilation, labeling, and secure containment to prevent accidental exposure or spills. - Provide chemical overflow pallets and store chemical containers by placing them on them. - Develop and implement a spill response plan, as part of the Emergency Response Plan, including containment measures, chemical spill/leak response drills, cleanup procedures, hazardous substance disposal, and emergency contact information. - Dispose of chemical waste according to regulations to prevent environmental contamination and worker exposure. - Regularly inspect and maintain chemical handling equipment, storage areas, and PPE to prevent leaks or accidental releases. | Kayapınar Municipality |
|  | Gender-Based Violence (GBV); Sexual Exploitation and Abuse/Sexual Harassment (SEA/SH) on Employees; Gender Inequality | Employees | - Provide GBV and SEA/SH awareness sessions for the management teams of the construction contractor and consultants to promote understanding and accountability. - Conduct regular awareness meetings with workers to educate them on GBV and SEA/SH issues and the importance of respectful workplace conduct. - Ensure all workers receive training on recognizing, preventing, and responding to GBV and SEA/SH incidents. - Require all workers to review, sign, and adhere to a Code of Conduct that explicitly addresses unacceptable behaviors related to GBV and SEA/SH. - Implement a confidential and accessible grievance mechanism specifically designed to capture and address GBV and SEA/SH-related complaints in a timely manner. | Kayapınar Municipality |
| Resource Efficiency and Pollution Prevention and Management |  |  |  |  |  |
|  | Waste Management | Employees Communities Flora and fauna Soil, water resources | - Separate waste at the source into waste categories determined in Waste Management Regulation, establish temporary waste storage area - Place labeled bins for each type of waste at strategic locations on-site to ensure correct disposal by workers. - Implement practices to reduce waste generation by optimizing material use and reusing materials where possible. - Contract with local recycling facilities to ensure that recyclable materials (e.g., metals, paper, plastic) are properly processed. - Store waste in designated, secured areas to prevent littering, leaching, and environmental contamination. - Use leak-proof containers for hazardous or liquid waste and ensure they are adequately labeled. - Contract with licensed waste disposal companies to handle non-recyclable and hazardous wastes in accordance with Waste Management Regulation. - Track and document the disposal process to ensure compliance and accountability. - Conduct regular awareness sessions and training for workers on waste reduction techniques, proper disposal practices, and the importance of waste management. - Regularly monitor waste management practices, conduct site inspections, and assess waste volumes to identify areas for improvement. - Establish a reporting system to document waste types, quantities, and disposal methods. - Develop a comprehensive waste management plan that includes waste reduction targets, disposal methods, monitoring schedules, and assigned responsibilities for effective waste management throughout the sub-project. - Use containment systems for waste that poses spill risks, and keep spill kits accessible. Train staff on immediate spill response actions to prevent soil and water contamination. - Conduct maintenance tasks, such as oil changes and battery replacements, off-site; | Kayapınar Municipality |
|  | Electronic Waste Disposal | Employees Communities Flora and fauna Soil, water resources | - Contract with recycling facilities and/or manufacturers to ensure proper disposal or recycling of obsolete equipment; - Agreements will be set with e-waste recycling facilities to ensure responsible disposal of electronic waste from solar panels, inverters, batteries, etc. | Kayapınar Municipality |
|  | Water Use | Flora and fauna Soil, water resources | - Use water efficiently when cleaning solar panels to minimize water consumption and wastewater production. - Implement wiper cleaning using rubber blade water sprayers that require minimal water, promoting water conservation practices. | Kayapınar Municipality |
|  | Wastewater Management | Flora and fauna Soil, water resources | - Portable toilet will be placed on a level, solid surface. - Toilet will be located away from food and beverage preparation and rest areas. - Toilet will be secured against wind or tipping. - The interior of the toilets will be cleaned and disinfected regularly. - Toilet will be emptied periodically with a vacuum truck, depending on occupancy. - Soap, disinfectant, and paper towels will be placed inside. - Deodorizing chemicals will be added regularly. - Ventilation openings in the toilets will be kept clear. - The area surrounding the toilets will be made safe with non-slip material. - Employees will be trained on the proper and hygienic use of portable toilet. - Cleaning and maintenance procedures will be recorded. . | Kayapınar Municipality |
|  | Soil and Groundwater Contamination | Employees Communities Flora and fauna Soil and water resources | - Contain and clean up any oil, chemical, lubricant, or fuel spill immediately to prevent environmental contamination. - Implement spill prevention and response measures. Maintain spill containment and clean-up kits on-site. Ensure all spills are contained, cleaned, and disposed of by licensed waste management companies. - Conduct routine servicing of construction vehicles and equipment at designated off-site locations to minimize the risk of leaks or spills. - Collect and store waste oil securely for recycling or dispose of it through licensed waste vendors to ensure safe handling. - Provide adequate sanitary facilities, including toilets and showers, for the construction workforce. Ensure prompt repairs and maintenance in the event of any leaks or spills to maintain hygiene and safety standards. | Kayapınar Municipality |
|  | Hazardous Substances Management | Employees Communities Flora and fauna Soil and water resources | - Maintain a comprehensive record of the types, quantities, and properties of hazardous materials to be stored on-site. - Establish a designated storage area specifically equipped for the safe storage of hazardous and toxic materials. - Ensure all storage containers are clearly labelled with appropriate hazard warnings, safety information, and emergency contact details to facilitate proper handling and identification. All chemicals will be managed in accordance with their Material Safety Data Sheets (MSDS). - Utilize suitable containers, tanks, and bunding systems to contain hazardous materials and prevent spills, leaks, or releases. Implement secondary containment measures, such as berms, dikes, or containment basins, to capture any accidental releases. - Ensure adequate ventilation and venting systems are in place within storage areas to prevent the accumulation of hazardous vapours or gases. - Identify and safely remove hazardous materials, including lead-containing components from solar panels and electronic waste from inverters, following proper disposal protocols. - Implement appropriate containment and handling procedures to minimize the risk of spills or releases of hazardous substances during storage and handling. - Arrange for the proper disposal or recycling of hazardous materials through licensed facilities to ensure safe and compliant waste management. | Kayapınar Municipality |
| Community Health and Safety |  |  |  |  |  |
|  | Glare from Solar Panels which can be a Safety Hazard for Drivers, Pedestrians, and Nearby Residents, Particularly if it Impairs Visibility or Causes Discomfort | Communities | - Ensure correct orientation of solar panels to minimize glare and reduce potential impact on road safety near the solar plant. - Apply anti-glare coatings to panels where required to further mitigate glare and enhance road safety in the vicinity. | Kayapınar Municipality |
|  | Risks Related With Gender Based Violence (GBV) Sexual Exploitation Abuse / Sexual Harassment (SEA/SH) | Communities | - Deliver ethical rules and public communication training to all employees to prevent gender-based violence (GBV), harassment, and abuse in the workplace. - Require all workers to sign and adhere to a code of conduct that promotes respectful behaviour. - Conduct regular awareness-raising sessions on-site focused on GBV prevention and other relevant social issues. - Establish a grievance mechanism to receive and address complaints related to GBV and workplace misconduct. | Kayapınar Municipality |
|  | Impacts on Local Economy, Livelihood Sources and Employment | Communities | - Upgrade existing roads if necessary to the sub-project site to ensure they do not restrict access to grazing land for local livestock grazers. - Regularly engage with local communities and maintain a grievance mechanism to address community concerns and feedback. | Kayapınar Municipality |
|  | Impacts on Vulnerable and Disadvantaged Individuals and Groups | Communities | - Implement a recruitment policy that promotes non-discriminatory hiring, provides tailored training for vulnerable groups, and offers support services such as transportation or childcare. - Develop and execute Corporate Social Responsibility activities to benefit local communities, focusing on identified needs such as road improvements and utility enhancements. | Kayapınar Municipality |
|  | Security Personnel | Communities | - Kayapınar Municipality will ensure that security personnel have not been involved in past abuses and are adequately trained. Force will only be sanctioned in preventive or defensive circumstances in proportion to the threat and security will operate within the law; - The grievance mechanism will allow communities and workers to express concerns regarding security issues and behaviour of security personnel. | Kayapınar Municipality |
| Biodiversity Conservation and Sustainable Management of Living Natural Resources |  |  |  |  |  |
|  | Disturbance on Biodiversity | Flora and fauna | - Ensure proper maintenance of exclusion fencing around the site, utilizing wildlife-friendly designs that allow small animals, such as hedgehogs, to pass safely. - Implement appropriate signage and fencing to separate sub-project access roads from other areas, limiting personnel and vehicle access to designated zones. - No harm will be caused to any fauna species observed within or around the sub-project site. All activities will be carried out in accordance with the principle of biodiversity protection, and necessary measures will be taken to avoid any adverse impacts on fauna. Where necessary, species will be relocated to an alternative area or outside the project site | Kayapınar Municipality |
| Stakeholder Engagement and Information Disclosure |  |  |  |  |  |
|  | Insufficient Stakeholder Engagement Activities and Public Consultation. | Communities Construction workforce | - Create channels for interaction and communication with local communities, ensuring that engagement activities are scheduled at convenient times. - Conduct regular consultations with relevant authorities and local communities to discuss project management and gather feedback. | Kayapınar Municipality |

# Appendices

## Appendix-1. Site Map

## Appendix-2. Copies of Existing Permitting Documentation

## Appendix-3. Copies of Title Deed(s)

## Appendix-4. Photographic Log

| Photo No: 01 |  |
| --- | --- |
| Date: 09.02.2025 |  |
| Location: 106/21 |  |
| Details/Notes: |  |
| Photo No: 02 |  |
| Date: 09.02.2025 |  |
| Location: 106/21 |  |
| Details/Notes: |  |
| Photo No: 03 |  |
| Date: 09.02.2025 |  |
| Location: 106/21 |  |
| Details/Notes: |  |

## Appendix-5. Construction Notice Template

“Çevreye verdiğimiz rahatsızlıktan dolayı özür dileriz!”

KAYAPINAR BELEDİYESİ 360 kWe/435.6 kWp

Kayapınar Municipality GES ALTPROJESİ YAPIM İŞİ

İşin Süresi:

Şikâyet, istek, soru ve yorumlarınız için:

| YÜKLENİCİ ADI | : |  | Adres: Telefon: E-posta: İletişim Formu: |  |
| --- | --- | --- | --- | --- |
| İŞVEREN | : |  | Adres: Telefon: E-posta: İletişim Formu: |  |
| İller Bankası A.Ş. (İLBANK) |  |  | Adres: İller Bankası A.Ş. Emniyet Mahallesi, Hipodrom Caddesi No:9/21, Yenimahalle/ANKARA Telefon: 0 (312) 508 79 79 E-posta: bilgiuidb@ilbank.gov.tr İletişim Formu: https://www.ilbank.gov.tr/form/bilgiedinmeuluslararasi |  |

## Appendix-6. E&S Incident Notification Form

1) Incident Details

| Date of Incident: [Please insert] | Time of Incident: [Please insert] |  |
| --- | --- | --- |
| Location of the Incident: | [Please insert] |  |
| Sub-borrower Name: | [Please insert] |  |
| Date Reported to ILBANK: [Please insert] | Reported to ILBANK by: [Please insert] | Notification Method: [Please insert] |
| Date Reported to WB: [Please indicate] | Reported to WB by: [Please insert] | Notification Type: [Please insert] |
| Contractor Name: | [Please insert] |  |
| Sub-contractor name (please indicate if involved in the incident): | [Please insert] |  |

| 2) Type of incident (please check all that apply) |  |
| --- | --- |
| ☐ Fatality ☐ Lost time injury ☐ Child labor ☐ Forced labor | ☐ Environmental pollution incident ☐ Disease outbreaks ☐ Acts of violence/protest ☐ Other |

3) Description/Narrative of Incident

4) Actions taken to contain the incident

For incidents involving a Contractor: Name of Contractor: _______________________ Have the works been suspended? Yes ☐ No ☐

5) What support has been provided to affected people

[Please briefly describe]

6) Please provide the supporting documents for the incident, victims and involved persons

(e.g. copies of the social security registration records, victim and witness statements, notification to authorities, legal investigation reports, training records, photographs, etc.)

## Appendix-7. Simplified Labor Management Procedure

## Appendix-8. Roles and Responsibilities

| Party | Role | Key Responsibilities |
| --- | --- | --- |
| Sub-borrower |  |  |
| Kayapınar Municipality | Sub-borrower Management | - Hold ultimate responsibility for the E&S performance of the subproject to the satisfaction of the ILBANK, including the performance of subproject contractors throughout the sub-financing agreement life cycle. - Establish Project Implementation Unit (PIU) following the execution of sub-financing agreements to carry out operational and administrative tasks to oversee the implementation of the E&S instruments and monitoring progress; allocate resources for the recruitment of in-house environmental, social and OHS staff under the PIU - Ensure that E&S instruments and procedures required by ILBANK is prepared within the timeframes agreed with ILBANK and allocate adequate financial and human resources – either from the Sub-borrower’s own resources or from the Subproject loan and implement. - Cooperate with the ILBANK representatives to discuss and agree on the ESAP and other E&S covenants for incorporation into sub-financing agreements to be executed between the ILBANK and the sub-borrower (with support from RD E&S team as necessary) - Ensure that EHSS requirements of ILBANK are incorporated into relevant contractor tender and agreement documents to be prepared in collaboration with the construction supervision consultant - Hold and use the authority and responsibility to stop any subproject related work activity if it poses an imminent danger to health, safety, or the environment. - Allocate resource to ensure monitoring of subproject E&S performance and reporting to ILBANK at IFI standards in line with the sub-financing agreement conditions - Facilitate monitoring visits and audits by ILBANK and their consultants - Notify the ILBANK RD – E&S Teams of any significant E&S incident or accident within maximum 24 hours of the accident/incident; contractually require the supervision consultants and/or contractors to promptly report such incident and accidents (timeframe to be defined by ILBANK) - Prepare and submit a detailed E&S Incident Investigation Form, supplemented by an RCA to be conducted pursuant to GIIPs, to ILBANK within 15 days of the accident/incident date for significant accidents or incidents (in line with the template presented in the E&S Supervision, Monitoring and Reporting Procedure). The investigation will be supplemented by a Root Cause Analysis (RCA). - Prepare and submit semiannual E&S monitoring reports to ILBANK. |
|  | E&S Team - Environmental staff - Social staff - OHS staff | - Participate in the training to be organized by ILBANK as part of ILBANK ESMS Training Procedure implementation - Ensure that E&S documentation required by ILBANK is prepared by qualified independent specialists and submitted to ILBANK for appraisal and credit. - Provide ILBANK with relevant adequate information to undertake the E&S due diligence in accordance with the ESMS (e.g. duly completed sub-borrower questionnaire and supporting documentation to be requested by ILBANK in accordance with the E&S Screening and Risk Classification and ESDD procedures) - Support the sub-borrower management as required in the review and evaluation of ESAP and other E&S covenants for incorporation into sub-financing agreements to be executed between the ILBANK and the sub-borrower - Ensure compliance of subprojects operations (including contractor activities on site) with national legislation and E&S requirements of the lending WB as included in the sub-financing agreements, ESAP and subproject-specific E&S documentation. - Undertake monitoring of subproject E&S performance and reporting to ILBANK at WB standards in line with the sub-financing agreement conditions - Ensure implementation of corrective actions in case of E&S non-compliances in coordination and agreement with ILBANK DG and RD E&S teams over reasonable timeframes - Coordinate the construction supervision consultants, contractors and/or external E&S consultants for collection of the monitoring data and compilation of or providing input to periodic monitoring reports as necessary and appropriate - Allow ILBANK representatives (including individual consultants) to access subproject facilities and records |
| Construction Supervision Consultants (“Müsavir”) | Management and E&S staff | Carry out the following tasks on behalf of the sub-borrowers: - Participate in the training sessions to be organized by sub-borrowers in line with the requirements of ILBANK ESMS Training Procedure - Supervise the construction works of contractors on-site, including implementation of subproject-specific E&S requirements by contractors on a daily basis - Ensure sufficient E&S capacity for implementation of E&S requirements as set out in the sub-financing agreements between the sub-borrower and ILBANK - Support the sub-borrowers for the supervision and review of E&S management documentation prepared by construction contractors and submit them to sub-borrowers upon finalization - Review monthly self-monitoring reports prepared by the construction contractors for early identification of E&S issues and/or non-compliances and submit them to municipalities/municipal utilities upon finalization - Prepare and submit regular monthly reports to Sub-borrower on the environmental, social and OHS issues of the Sub-Project during the construction phase - Identify E&S non-compliances on site and enforce construction contractors to undertake corrective actions within defined and agreed timeframes - Support the sub-borrowers (as requested) in the preparation of periodic E&S monitoring reports to be submitted to ILBANK in line with the ILBANK E&S Supervision, Monitoring and Reporting Procedure - Notify the sub-borrower of any significant E&S incident or accident that have taken place in subproject related operations within 24 hours. |
| Construction Contractor | Management and E&S staff | - Ensure sufficient E&S capacity for implementation of E&S requirements as set out in the construction contracts - Participate in the training sessions to be organized by sub-borrowers in line with the requirements of ILBANK ESMS Training Procedure - Prepare subproject-specific E&S management plans and procedures prior to start of construction works as required by the construction contracts - Comply with the requirements of national legislation and implement the E&S requirements as set out in the sub-financing agreements (executed between ILBANK and the sub-borrowers) and construction contracts - Submit periodic (in frequencies to be set by ESAP) E&S self-monitoring reports to the municipalities/municipal utilities through construction supervision consultants (“müşavir”) – in line with the format provided by ILBANK. - Fill in monthly occupational health and safety (OHS) forms – reviewed by construction supervision consultants. - Implement corrective actions in case of E&S non-compliances under the supervision of sub-borrower’s construction supervision consultant - Promptly notify the sub-borrower of any significant E&S incident or accident that have taken place in subproject related operations (timeframe to be defined by ILBANK no later than 24 hours). |

## Appendix-9. Chance Find Procedure

Introduction

This document describes the Chance Find Procedure for subproject, outlining the procedures that will be followed in case of chance finds occur during the construction activities associated with the subproject.

Scope

This Chance Find Procedure (CFP) will be implemented for Kayapınar Municipality sub-project in order to manage any chance finds that may be encountered during the construction activities. The purpose of the CFP document is to:

- outline the applicable legislation and standards relevant to this procedure;

- define roles and responsibilities;

- define project commitments, operational procedures, training requirements and guidance relevant to this procedure; and

- define monitoring and reporting procedures.

Although there are no known archaeological sites or remains within the subproject area, it is considered that there may be a potential to encounter archaeological findings during the construction of the subproject. Activities which have high potential to lead to discover or adversely affect the archeological resources are;

- topsoil stripping

- excavation and earthworks

This CFP is prepared in order to provide information to the contractors and employees regarding the actions to be taken in case of an archaeological chance find discovery.

Legislation and Standards

Legislation and standards that apply to the project comprise the following:

- Word Bank Environmental and Social Standard (ESS) 8: Cultural Heritage

- applicable Turkish laws and national standards

- other commitments to and requirements of Turkish government authorities

- other industry guidelines with which the project has committed to comply

In Turkey, movable and immovable cultural and natural assets are protected and preserved by the Law on Preservation of Cultural and Natural Assets (Law No. 2863) published in the Official Gazette dated 23.07.1983 and numbered 18113. Law 2863 establishes legal protection for the following:

- all natural assets and immovable cultural assets constructed up until the end of the 19th century,

- any immovable cultural asset from after the end of the 19th century, identified by the Ministry of Culture and Tourism as an important asset worthy of preservation,

- all immoveable cultural assets located within archeological sites,

- buildings/areas that have witnessed significant historical events during the National War and the foundation of the Turkish Republic and dwellings that have been used by Mustafa Kemal ATATÜRK, regardless of time and registration.

The Ministry of Culture and Tourism is the responsible body to take decisions for protection of cultural heritage in Turkey at the national level. As part of the Ministry, the High Commission for the Protection of Cultural Assets is responsible for protecting and restoring immovable cultural assets. Implementation of the decisions and regulations issued by the Ministry are undertaken by local administrations. At local level, there are Cultural Assets Protection Regional Boards defined by the Ministry of Culture and Tourism, which are responsible for preservation, registration and classification of cultural heritage within their respective jurisdictions. The relevant Regional Board for the project is the Batman Cultural Heritage Protection Regional Board Directorate.” According to Law 2863, all the natural and cultural assets qualified for legal preservation are properties of the State. Therefore, regional boards have the power and authority to provide legal protection to the preservation sites and to approve or reject all the activities, which have potential negative impacts on the preservation sites such as construction, demolition and excavation activities.

Roles and Responsibilities

Principal roles and responsibilities for the implementation of this procedure are outlined below.

| Role | Responsibilities |
| --- | --- |
| Contractor -Project Manager | - Overall responsibility for the development, review, approval and coordination of the numerous activities required to initiate, conduct and complete construction. - Ensure that this procedure is prepared, and updated as required, based on the activities undertaken as part of the project. - Ensure that adequate resources are made available to implement the procedures and guidelines outlined in this procedure. |
| Contractor -Environmental and Social (E&S) Expert | - Initiation, development, implementation and coordination of the CFP during construction. - Ensure that adequate training is given to all site personnel and sub- contractors, covering the procedures and guidelines outlined in this procedure. Establish appropriate control procedures and conduct audits as necessary. - Consultation with and reporting to relevant government bodies in case of potential archeological chance finds. - Record all confirmed chance finds by filling up the “Chance Find Reporting Form” and maintain copies in a log-book. Ensure that the chance finds log-book is up to date. |
| Contractor - Site Manager | - Day-to-day implementation of the provisions of the CFP in the field during construction. - Notify the E&S Expert regarding potential chance finds during construction. |
| Employees | - Understand and comply with archeological chance finds procedures and guidelines outlined in this procedure. - Reporting of the potential chance finds to the Site Manager. |

Impact Avoidance and Mitigation

In the event of an archaeological discovery, the following actions will be implemented:

- All staff involved in land clearance and excavation activities will take the responsibility for managing archaeological protection and will be trained in these aspects by the E&S Expert.

- In case any potential chance find is encountered, all construction activities will cease immediately in the vicinity of the chance find.

- The Site Manager will be contacted immediately. The discovered site location, the characteristics of the potential archaeological material and photos will be recorded by the Site Manager, who in turn will inform the E&S Expert.

- Batman Museum Directorate will be notified at the latest within three days after the chance find is encountered. Contact details of the Batman Museum Directorate are given below:

Address: Yeni Kültür Merkezi, Belde Mahallesi, Barış Bulvarı No:129

Telephone: 0 488 214 10 51

E-mail: batmanmuze@kultur.gov.tr

- The site and its vicinity will be secured 24 hours a day against damage or loss, until inspection by the authority.

- The E&S Expert will fill up a “Chance Find Report Form” for each confirmed chance find and inform the Project Manager about the date that the construction work can resume, which is determined by the authorities concerning the conservation of the heritage.

- Further steps to be followed and proper plan to be implemented for the management of the finds (Changes in the layout, conservation, preservation, restoration and salvage) will be decided and reported in writing by the authorities in charge.

- Photographs of the potential artifacts that are likely to be encountered in the construction site are presented in the following pages to be used during the training of the relevant staff.

Verification and Monitoring

E&S Expert/s will record all cases of archaeological chance finds. He/she will fill up a “Chance Find Reporting Form” for each chance find confirmed by the authority and maintain copies in a logbook. A sample of a reporting form which can be used to record chance finds is included below. The chance find logbook will be summarized on an annual basis and records included in semi-annual monitoring reports to verify that correct management procedures have been followed. Action items will be taken in cases of non-adherence to this CFP.

Reporting

Contractor will comply with reporting requirements including chance finds defined in site-specific ESMP (contractor will develop monthly and quarterly monitoring reports and submit to Kayapınar Municipality through supervision consultant; Kayapınar Municipality will examine submit the reports to ILBANK quarterly (and monthly if requested by ILBANK); ILBANK will inform the World Bank by providing regular semi-annual monitoring reports.

| Kayapınar Municipality 360 kWe / 435.6 kWp SPP Subproject Chance Find Reporting Form |  |  |  |
| --- | --- | --- | --- |
| REGISTRATION |  |  |  |
| Name of recorder: |  |  |  |
| Date and time of discovery: |  |  |  |
| Site Name: | Coordinates X Y |  |  |
| Description of find: |  |  |  |
| Photograph: |  |  |  |
| Estimated weight and dimensions: |  |  |  |
| CONTACT PERSON |  |  |  |
| Name/Title/Duty: |  |  |  |
| Date and Time: |  |  |  |
| Contact information: |  |  |  |
| Details of conversation: |  |  |  |
| DECISIONS |  |  |  |
| Any protection measures to be implemented: |  |  |  |
| Movable or immovable: If moved, please specify the new location. |  |  |  |
| Further actions required: |  |  |  |
| Recommence date and time: |  |  |  |
| Notes: |  |  |  |
| SUBMISSION |  |  |  |
| Name: |  | Date: |  |

## Appendix-10. Change Notification Form

| Change Notification Form |  |  |
| --- | --- | --- |
| Subproject Name |  |  |
| Subproject Location |  |  |
| Subproject Phase | ☐ | Pre-construction |
|  | ☐ | Construction |
|  | ☐ | Operation |
| Name of the Institution Notifying the Change |  |  |
| Date |  |  |
| Category of the Change (please select all that apply) | ☐ | Legislative Change |
|  | ☐ | Design Change |
|  | ☐ | Schedule Change due to E&S factors |
|  | ☐ | Project Schedule Changes due to technical, financial, legal or administrative factors |
|  | ☐ | Changes due to E&S issues encountered at subproject implementation |
|  | ☐ | Contractor or Construction Supervision Consultant Change |
|  | ☐ | Other (please specify below) |
| Detailed Description of the Change(s) |  |  |
| Documents Submitted with Change Notification Form |  |  |
| Name of the Staff Notifying the Change |  |  |
| Position of the Staff Notifying the Change |  |  |
| Signature |  |  |

## Appendix-11.  Emissions and Environmental Noise Calculations

Air Quality/Emission

Air pollution will mainly originate from dust emissions and exhaust emissions as well as Greenhouse Gas (GHG) emissions. Considering the location of the sub-project area, sensitive receptors are not expected to be affected. During the construction phase of the sub-project, the impacts on air quality will mainly originate from dust, exhaust and greenhouse gas emissions:

• Dust emissions during site preparation, excavation, filling and compaction works carried out for construction works.

• Dust emissions from vehicle movements for transporting various construction materials to the project site.

• Exhaust emissions from vehicles used in construction activities.

• Greenhouse gas emissions from small amounts of vehicles and machinery.

Since a limited number of equipment and machinery will be operating on the sites, these air quality impacts will be limited to the area and in the short term. In addition, the recycling wastewater distribution network will follow the cadastral roads and the construction will be carried out in stages. Therefore, the receivers will be limited to those located near the construction sites.

Calculation of dust emission topsoil stripping

In the calculation of the dust emissions to be generated, the emission factors given in Table 2.7 of the “Regulation on Control of Industrial Air Pollution” (Amended Table: RG-20.12.2014-29211) published in the Official Gazette dated 03.07.2009 and numbered 27277 were used and the results were evaluated within the framework of the same regulation.

The calculations were made using both “uncontrolled” emission factors, considering that the most adverse conditions could occur during dust formation, and “controlled” emission factors, assuming that the necessary control measures were taken.

The area where the SPP project site will be established is 4.650 m2. In this area, 10 cm topsoil stripping will be used to strip 465 m3 of soil.

(Soil Bulk Density is taken as 1.6 tons/m3)

465 m3* 1.6 tons/m3=744 tons

Daily working time is planned as 8 hours. Excavation work is planned as 192 hours in total.

744 tons/192 hours= 3.9 tons/h

Table 1. Control of Industrial Air Pollution

| Sources | Uncontrolled | Controlled | Unit |
| --- | --- | --- | --- |
| Removal | 0.025 | 0.0125 | kg/ton |
| Loading | 0.0100 | 0.005 |  |
| Unloading | 0.010 | 0.005 |  |
| Transportation (total round trip distance) | 0.7 | 0.35 | kg/km-vehicle |
| Storage | 5.8 | 2.9 | Dust/ha-day |

Mass Flow Rate of Dust Emission to Occur During Removal, Loading and Unloading of topsoil

Uncontrolled; E1 = 3.9 tons/hour x (0.025+0.01+0.01) kg/ton = 0.175 kg/hour

Controlled; E1 = 3.9 tons/hour x (0.0125+0.005+0.005) kg/ton= 0.08775 kg/hour

Mass Flow Rate of Dust Emission to Occur During the Transportation of Topsoil

Topsoil taken from the field during construction work will be temporarily stored in the topsoil storage area that will also be located within the work area; this distance is an average of 0.3 km round trip. Assuming that each truck used during transportation can carry 25 tons of material and therefore will make 1 trip in approximately 1 working day (25 tons/23.32 tons/hour), the mass flow rate of dust emissions that will occur during transportation is;

Uncontrolled; E2 = (0.7 kg/km.vehicle) x (0.3 km/1 trip/vehicle) x (1 trip/1 hour) = 0.21 kg/hour

Controlled; E2 = (0.35 kg/km.vehicle) x (0.3 km/1 trip/vehicle) x (1 trip/1 hour) = 0.11 kg/hour

Dust Emission Mass Flow Rate to be Formed During the Storage of Vegetal Soil

Uncontrolled; E3 = (5.8 kg/ha-day)x(1 ha/8 weeks/ 6 days/week/8 hours/day)= 0.0152 kg/hour

Controlled; E3 = (2.9/ha-day)x(1 ha/8 week/6 days/week/8 hours/day)= 0.0076 kg/hour

Accordingly, the total mass flow rate of dust emission to be formed from the stripping operations of the vegetal soil to be carried out;

Uncontrolled; ETOTAL-1 = 0.175 kg/h + 0.21 kg/h + 0.0152 kg/h ≈ 0.386 kg/h

Controlled; ETOTAL-1 = 0.08775 kg/h + 0.11 kg/h + 0.0076 kg/h ≈ 0.20 kg/h

When calculating the dust emission to be generated during the vegetative soil stripping operations, it was taken into account that the works would be carried out under the most adverse conditions. As stated in the “Regulation on Control of Industrial Air Pollution; for newly established facilities, “Calculation of the Contribution Value to Air Pollution” is required if the pollutant mass flow rates are exceeded.

Considering that all the works to be carried out within the scope of the vegetal soil stripping operations to be carried out at the construction site will be carried out in the same time period (worst case scenario), the dust emission to be generated has been calculated as 0.40 kg/hour for the uncontrolled case and 0.20 kg/hour for the controlled case. Therefore, as stated in “Regulation on Control of Industrial Air Pollution”; since the specified pollutant mass flow rates are not exceeded for the topsoil stripping operation, it has not been deemed necessary to calculate the “Contribution Value to Air Pollution” using an internationally accepted distribution model in the facility area of influence.

The construction equipment and transportation vehicles in question will be used at different times during the day.

Calcutation of dust emission Excavation Soil

Within the scope of the sub-project, excavation works will be carried out for 20 meters long ETL. The volume of the area to be excavated;

20 m * 0.8 m * 1 m = 16 m3

(Soil Volume Weight is taken as 1.6 tons/m3)

16 m3 * 1.6 tons/m3 = 25.6 tons

Daily working time is planned as 8 hours. Excavation work is planned as 192 hours in total.

25.6 tons/192 h = 0.533 tons/h

Mass Flow Rate of Dust Emission to Occur During Removal, Loading and Unloading of Excavation Soil

Uncontrolled; E1 = 0.533 tons/hour x (0.025+0.01+0.01) kg/ton = 0.024 kg/hour

Controlled; E1 = 0.533 tons/hour x (0.0125+0.005+0.005) kg/ton= 0.012 kg/hour

Mass Flow Rate of Dust Emission to Occur During the Transportation of  Excavation Soil

Topsoil taken from the field during construction work will be temporarily stored in the excavation soil storage area that will also be located within the work area; this distance is an average of 0.3 km round trip. Assuming that each truck used during transportation can carry 25 tons of material and therefore will make 1 trip in approximately 1 working day (25 tons/20.98 tons/hour), the mass flow rate of dust emissions that will occur during transportation is;

Uncontrolled; E2 = (0.7 kg/km.vehicle) x (0.3 km/1 trip/vehicle) x (1 trip/1 hour) = 0.21 kg/hour

Controlled; E2 = (0.35 kg/km.vehicle) x (0.3 km/1 trip/vehicle) x (1 trip/1 hour) = 0.11 kg/hour

Dust Emission Mass Flow Rate to be Formed During the Storage of Excavation Soil

Uncontrolled; E3 = (5.8 kg/ha-day)x(1 ha/1 weeks/ 6 days/week/8 hours/day)= 0.12 kg/hour

Controlled; E3 = (2.9/ha-day)x(1 ha/1 week/6 days/week/8 hours/day)= 0.06 kg/hour

Accordingly, the total mass flow rate of dust emission to be formed from the stripping operations of the excavation soil to be carried out;

Uncontrolled; ETOTAL-1 = 0.024 kg/h + 0.21 kg/h + 0.12 kg/h ≈ 0.354 kg/h

Controlled; ETOTAL-1 = 0.012 kg/h + 0.11 kg/h + 0.06 kg/h ≈ 0.192 kg/h

When calculating the dust emission to be generated during the excavation soil operations, it was taken into account that the works would be carried out under the most adverse conditions. As stated in the “Regulation on Control of Industrial Air Pollution; for newly established facilities, “Calculation of the Contribution Value to Air Pollution” is required if the pollutant mass flow rates are exceeded.

Considering that all the works to be carried out within the scope of the excavation soil operations to be carried out at the construction site will be carried out in the same time period (worst case scenario), the dust emission to be generated has been calculated as 0.354 kg/hour for the uncontrolled case and 0.192 kg/hour for the controlled case. Therefore, as stated in “Regulation on Control of Industrial Air Pollution”; since the specified pollutant mass flow rates are not exceeded for the topsoil stripping operation, it has not been deemed necessary to calculate the “Contribution Value to Air Pollution” using an internationally accepted distribution model in the facility area of influence.

The construction equipment and transportation vehicles in question will be used at different times during the day.

It will be used in soil filling materials and leveling works that occur during excavation works.

Emission calculation from vehicles

The provisions of the Exhaust Gas Emission Control and Gasoline and Diesel Quality Regulation, which was published in the Official Gazette dated 30.11.2013 and numbered 28837 and entered into force, and the Exhaust Gas Emission Control Regulation, which was published in the Official Gazette dated 11.03.2017 and numbered 30004, shall be complied with.

During construction, the fuel to be spent is only necessary for the work machines to be used, there will be no fuel consumption for heating etc. The usage periods and fuel consumptions of the work machines to be used during the construction phase of the business are shared in Table 2.

Table 2. Usage periods of the work machines to be used in the facility

| Machine type | Number | Power (hp/h) | Working Time (h/day) |
| --- | --- | --- | --- |
| Crane | 1 | 200 | 8 |
| Excavator | 1 | 200 | 8 |
| Truck | 1 | 200 | 8 |
| Pile Driver | 1 | 90 | 8 |
| Water Tank | 1 | 120 | 8 |

The fuels to be used in the land preparation and construction phase of the sub-project will be diesel fuel to be used during the work of the construction equipment. Apart from this, there is no other type of fuel to be used in the sub-project. Diesel fuel will be preferred as fuel for the construction equipment to be used within the scope of the sub-project. There will be no fuel storage in the sub-project area and the fuel supply to the construction equipment will be made with fuels supplied from authorized stations. The characteristics of diesel fuel are given below:

Table 3. Diesel Properties

| Properties | Diesel | Properties | Diesel |
| --- | --- | --- | --- |
| Consistency | Very fluid | Carbon Wastes (%) | Trace |
| Type | Distilled | Sulfur (%) | 0.4-0.7 |
| Color | Amber | Oxygen-Nitrogen (%) | 0.2 |
| Density (150c-gr/cm3) | 0.8654 | Hydrojen (%) | 12.7 |
| Viscosity (380 oC) | 2.68 | Carbon (%) | 86.4 |
| Pour Point (0oC) | -18 | Water and Sediment (%) | Trace |
| Atomization Temperature (0oC) | Atmospheric | Ash (%) | Trace |
| Pumping Temperature (0oC) | Atmospheric | Heat Value | 9.387 |

Source: Air Pollution Control and Supervision, Chamber of Chemical Engineering, May, 1999

The emission factors table determined by the EPA (Environment Protection Agency) was used for the construction equipment to be used within the scope of the sub-project.

Table 4. Emission Factors Used in Calculations

| Power | Year | CO (g/kWh) | HC (g/kWh) | NOx (g/kWh) | PM (g/kWh) |
| --- | --- | --- | --- | --- | --- |
| 56 ≤ kW < 130 (75 ≤ kW <175) | 2012 and above | 5,0 | 0,19 | 0,40 | 0,02 |
| 130 ≤ kW < 560 (175 ≤ kW <560 | 2011 and above | 3,5 | 0,19 | 0,40 | 0,02 |

Source: USEPA Standards

Using the data in the table above, exhaust gas emissions that will occur during the construction and operation phases are calculated with the formula below and entered into the tables.

Emission Value (kg/h) = Emission Factor x Engine Power (kW) x Number x kg/1000 gr

Table 5. Emission calculations

| Equipment to be used | Piece | Hp | kW | Emission Factor (g/kWh) |  | Emission Value (kg/sa) |
| --- | --- | --- | --- | --- | --- | --- |
| Excavator | 1 | 200 | 149 | CO | 3,5 | 0,52 |
|  |  |  |  | HC | 0,19 | 0,03 |
|  |  |  |  | NOx | 0,4 | 0,06 |
|  |  |  |  | PM | 0,02 | 0,003 |
| Crane | 1 | 200 | 149 | CO | 3,5 | 0,52 |
|  |  |  |  | HC | 0,19 | 0,03 |
|  |  |  |  | NOx | 0,4 | 0,06 |
|  |  |  |  | PM | 0,02 | 0,003 |
| Pile Driver | 1 | 90 | 67.05 | CO | 5 | 0,34 |
|  |  |  |  | HC | 0,19 | 0,013 |
|  |  |  |  | NOx | 0,4 | 0,026 |
|  |  |  |  | PM | 0,02 | 0,0013 |
| Truck | 1 | 200 | 149 | CO | 3,5 | 0,52 |
|  |  |  |  | HC | 0,19 | 0,03 |
|  |  |  |  | NOx | 0,4 | 0,06 |
|  |  |  |  | PM | 0,02 | 0,003 |
| Water Tanker | 1 | 120 | 89.5 | CO | 5 | 0,4475 |
|  |  |  |  | HC | 0,19 | 0,017 |
|  |  |  |  | NOx | 0,4 | 0,036 |
|  |  |  |  | PM | 0,02 | 0,002 |

1 Hp = 0.745 kW.

When emissions from all vehicles are added together;

Table 6. Amount of Emission

| Pollutant | Amount (kg/h) | Working Time (h) | Total Amount (kg/8 h) | 24 hour emissions |
| --- | --- | --- | --- | --- |
| CO | 2.3475 | 8 | 18.78 kg | 18.78 kg/24 h= 0.7875 kg/h |
| HC | 0.12 | 8 | 0.96 kg | 0.96 kg/24 h = 0.04 kg/h |
| NOx | 0.242 | 8 | 1.936 kg | 1.936 kg/24 h = 0.08 kg/h |
| PM | 0.0123 | 8 | 0.0984 kg | 0.0984kg/24 h = 0,004 kg/h |

The calculation was made assuming that all vehicles were operating at maximum operating time and in the same month.

| Pollutant | Amount (kg/h) | Mass flow rate (kg/hour) given in Annex-2 Table 2.1 of the “Regulation on Control of Air Pollution from Industrial Sources” | Evaluation |
| --- | --- | --- | --- |
| CO | 0.7875 | 50 | Below the limit value |
| HC | 0.04 | 2 | Below the limit value |
| NOx | 0.08 | 4 | Below the limit value |
| PM | 0.004 | 1 | Below the limit value |

The calculated exhaust gas emission amounts were calculated cumulatively assuming that all machinery and equipment operate at the same time and are entered in the table above. When the calculated hourly mass flow rate (kg/hour) value was compared with the mass flow rate (kg/hour) values given in Annex-2 Table 2.1 of the “Regulation on Control of Industrial Air Pollution”, it was seen that the emission mass flow rates were below the limit values given in the regulation. The calculations were made based on the assumption that all work machines operate simultaneously and continuously in their areas of use, and in reality, such an application is not very possible. Therefore, the emission levels that will occur in reality will be lower than the emission levels found in the calculations.

Where the requirements in Türkiye differ from the levels and measures presented in the EHS Guidelines, the more stringent (such as the most stringent discharge and emission standards) will be applied in the project specification.

Noise

The sub-project activities are planned to be completed in ~2 month. Within the scope of the sub-project, work will be carried out during the daytime, 6 days a week, 8 hours a day.

The sound power levels of the equipment were calculated according to the formulas given below according to the permitted sound power levels defined in the table given in Article 5 of the “Regulation on Noise Emission in the Environment Created by Equipment Used in Open Areas”, which was published in the Official Gazette dated 30.12.2006 and numbered 26392 and entered into force, and data from similar activities were also taken into account.

Table 7. Equivalent Noise level to the distances According to Distribution

| Distance (m) | 40 | 50 | 100 | 200 | 300 | 400 | 500 | 750 | 1000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Equivalent noise level | 64.4 | 62.3 | 56.0 | 49.3 | 45.3 | 42.4 | 40.1 | 35.8 | 32.8 |

Since the closest mosque, vineyard, garden house to the sub-project area is 100 meters away, it has been determined that it will remain below the limit values ​​specified in the Environmental Noise Control Regulation published in the official gazette dated 30.11.2022 and numbered 32029.

Table 8. Environmental Noise Level Limit Values (Environmental Noise Control Regulation)

| Noise Source | Measured Parameter | Environmental Noise Level |  |  |
| --- | --- | --- | --- | --- |
|  |  | Daytime (07:00 - 19:00) | Evening (19:00 - 23:00) | Night (23:00 - 07:00) |
| Industrial facilities transportation resources | LAeq,5min. | 65 dB(A) | 60 dB(A) | 55 dB(A) |
| Workplaces(2) | LAeq,5min. | Background + 5 dB(A) |  | Background + 3 dB(A) |
| In case of more than one workplace | LAeq,5min. | Background + 7 dB(A) |  | Background + 5 dB(A) |
| All sources | LCmax | 100 dB(C) |  |  |

- : These limit values are valid as of 31.12.2023. These limit values are valid for each 1/3 octave of the specified frequency range band. In the acoustic reports prepared until this date, environmental noise measurement results and measurement results measures identified are included.

- : Each workplace contributing to the background noise level is jointly responsible for meeting this limit value. Each workplace takes necessary measures according to their contribution to noise.

Table 8. IFC General EHS Guides Noise Levels

| Buyer | Daytime (07:00 - 22:00) | Night (22:00 - 07:00) |
| --- | --- | --- |
| Settlement Areas | 55 | 45 |
| Commercial/industrial areas | 70 | 70 |

Although the limit value meets the limits of the relevant national regulation, it is above the IFC standards. The calculations were made assuming that all equipment will operate simultaneously. In real life, lower environmental noise levels are expected. In addition, in case of any complaints about noise, measurements will be taken to determine the environmental noise level caused by construction work and if it is high, additional measures such as barriers, arrangement of working hours, etc. will be taken.

## Appendix-12.  Minutes of Public Consultation Meeting

Türkiye Public and Municipal Renewable Energy Project (PUMREP)

KAYAPINAR MUNICIPALITY

SOLAR POWER PLANT PROJECT

Minutes of Public Consultation Meeting

Meeting Date: 27.10.2025

Meeting Time: 14:00

Meeting Venue: Kayapınar Municipality Meeting Hall

PUBLIC CONSULTATION

Within the scope of the sub-project, the Environmental and Social Management Plan Checklist (ESMP-Checklist) and the Stakeholder Engagement Plan (SEP) have been prepared in accordance with Turkish environmental and social legislation, as well as the World Bank Environmental and Social Standards, Safeguard Policies, World Bank General EHS Guidelines, Industry Sector Guidelines, and İLBANK’s PUMREP Environmental and Social Management System (ESMS) and Stakeholder Engagement Plan (SEP).

As part of the stakeholder engagement process, a Public Consultation Meeting was held on 27 October 2025 at 14:00 in Kayapınar Municipality Meeting Hall. To inform the local community about the meeting, printed materials such as brochures and posters were prepared and displayed, and announcements were published on the Kayapınar Municipality website as well as in local and national newspapers.

## Meeting Summary

At the Kayapınar Municipality Public Consultation Meeting, a representative from the consulting firm provided detailed information about the process and content of the reports prepared for the implementation of the subproject. A presentation was made outlining the benefits the subproject will provide to the municipality and the local community. The presentation is included in Annex-6.

The meeting was attended by 1 municipal staff members, 17 local residents, making a total of 18 participants.

The area where the sub-project will be established (as a neighborhood, block and parcel), project power, equipment and technical specifications to be used, annual production of the project were specified and information was given that the legislative obligations were fulfilled.

Within the scope of the Public Consultation Meeting; the consultant company environmental expert conveyed the environmental and social risks of the sub-project from the content of the ESMP Report, the works planned to prevent the said risks, the effects of the geographical location of the region where the project will be located and the climate conditions on the project and the analyses made on possible natural disasters. In addition, during the Public Consultation Meeting, information was provided to the participants regarding the grievance channels that can be used to submit any complaints, concerns, suggestions, or opinions during the pre-construction, construction, and operation phases of the project. In line with the information provided, the meeting was concluded with questions and answers and lasted approximately 45 minutes.

## Question and Answer Section

| Question 1. |  |
| --- | --- |
| Name / Occupation | Yenice Neighborhood Resident |
| Will we be able to sell the electricity we produce? |  |
| Answer 1. |  |
| Name / Occupation | CA Engineering |
| The energy generated by the project will first be utilized to meet the water demand. Once the water needs have been fully satisfied, the process of selling the surplus energy can begin. |  |

## Meeting Conclusion

The Public Consultation Meeting, which lasted approximately 45 minutes, included a presentation by representatives of the consulting firm providing information about the project. Following the presentation, a question-and-answer session was conducted to gather participants’ views and inquiries. During the meeting, information was shared regarding the environmental, social, and economic aspects of the Kayapınar Municipality PV Project, as well as the next phase of the sub-project. In addition, participants were informed about the grievance mechanism available for submitting complaints, requests, or suggestions throughout the project duration. The meeting concluded after collecting participants’ feedback and recommendations.

## Participant List

# ATTACHMENTS

## Annex-1: Photo of the Public Consultation Meeting (27.10.2025)

## Annex-2: National Newspaper Advertisement (Newspaper Announcement Date –Takvim 16.10.2025)

## Annex-3: Local Newspaper Advertisement (Newspaper Advertisement Date – GercüşHaber Newspaper -16.10.2025)

## Annex-4: Kayapınar Municipality Announcements (16.10.2025)

## Annex-5: Kayapınar Municipality Public Consultation Meeting Brochure

## Annex-6: Public Consultation Meeting Presentation

https://www.ifc.org/en/insights-reports/2000/general-environmental-health-and-safety-guidelines

Sensitive complaints could include the following (not an exhaustive list): 1) Sexual exploitation and/or any type of abuse by a staff member; 2) Fraud and/or corruption by a staff member, such as involvement in bribery or misusing funds; 3) Any action which constitutes a breach of ILBANK code of conduct including staff behavior..

Any incident or accident relating to the subproject which has, or is likely to have, a significant adverse impact on the environment and/or health and safety of communities or employees (direct or contracted) involved in the subprojects related operations will be considered significant, including, inter alia, chemical and/or hydrocarbon materials spills; fire, explosion or unplanned releases, including during transportation; ecological damage/destruction; traffic or other type of accidents that could result in fatalities or serious injuries affecting employees and/or public complaint or protest; failure of emissions or effluent treatment; legal/administrative notice of violation; penalties, fines, or increase in pollution charges; negative media attention; chance cultural finds; labor unrest or disputes; local community concerns.

https://www.soilquality.org.au/factsheets/bulk-density-measurement

https://gsim2hwnpbvwtwmb1dg11z6.blob.core.windows.net/media/documents/8866271100_202404051549238_Product%20Information%20Sheet%20%28EU_2021_EP%29tr_TR.pdf

https://sbsolar.com.tr/1kw-kac-hp-bir-beygir-kac-kw?srsltid=AfmBOopeJLuU2e08CtSYKdRWghT6TSx7iJDNzzfTjy0U2vio8kOh7QKR